Pasadena Explosion

At approximately 1:00 p.m. on the 23rd October 1989 Phillips’ 66 chemical complex at Pasadena, near Houston (USA) experienced a chemical release on the polyethylene plant. A flammable vapor cloud formed which subsequently ignited resulting in a massive vapor cloud explosion. Following this initial explosion there was a series of further explosions and fires.

The consequences of the explosions resulted in 23 fatalities and between 230 – 300 people were injured.

Extensive damage to the plant facilities occurred.

The day before the incident scheduled maintenance work had begun to clear three of the six settling legs on a reactor. A specialist maintenance contractor was employed to carry out the work. A procedure was in place to isolate the leg to be worked on. During the clearing of No.2 settling leg part of the plug remained lodged in the pipework. A member of the team went to the control room to seek assistance. Shortly afterwards the release occurred. Approximately 2 minutes later the vapor cloud ignited.

The accident investigation established that the single isolating ball valve was actually open at the time of the release. The air hoses to the valve had been cross-connected so that the air supply that should have closed the valve actually opened it.